# Lab Title:Wireshark – DNS

MUHAMMAD RIZWAN KHALID (180459)

SYED ABBAS HUSSAIN NAQVI (177410)

BSCS-6A

Objective of this lab:

As described in Section 2.5 of the textbook, the Domain Name System (DNS) translates hostnames to IP addresses, fulfilling a critical role in the Internet infrastructure. In this lab, we’ll take a closer look at the client side of DNS. Recall that the client’s role in the DNS is relatively simple – a client sends a query to its local DNS server, and receives a response back. Much can go on“under the covers,” invisible to the DNS clients, as the hierarchical DNS servers communicate with each other to either recursively or iteratively resolve the client’s DNS query. From the DNS client’s standpoint, however, the protocol is quite simple – a query is formulated to the local DNS server and a response is received from that server.

Before beginning this lab, you’ll probably want to review DNS by reading Section 2.5 of the

textbook. In particular, you may want to review the material on local DNS servers, DNScaching, DNS records and messages, and the TYPE field in the DNS record.

Instructions:

- Read carefully before starting the lab.

- These exercises are to be done individually.

- You are supposed to provide the answers to the questions listed at the end of this document and upload the completed report to your course’s LMS site.

- Avoid plagiarism by copying from the Internet or from your peers. You may refer to source/ text but you must paraphrase the original work.

- Complete the lab half an hour before the lab ends.

- At the end of the lab, an online Quiz will be conducted to evaluate your understanding.

- Introduction to DNS

There are two ways to identify a host -- a hostname and an IP address.  People prefer the more mnemonic hostname identifier, while routers prefer fixed-length, hierarchically-structured IP addresses. In order to reconcile these different preferences, we need a directory service that translates hostnames to IP addresses. This is the main task of the Internet's Domain Name System (DNS).  The DNS is (i) a distributed database implemented in a hierarchy of name servers and (ii) an application-layer protocol that allows hosts and name servers to communicate in order to provide the translation service.

- Introduction to NSLOOKUP

In this lab, we’ll make extensive use of the nslookup tool, which is available in most Linux/Unix and Microsoft platforms today. To run nslookup in Linux/Unix, you just type the nslookup command on the command line. To run it in Windows, open the Command Prompt and run nslookup on the command line.

In it is most basic operation, nslookup tool allows the host running the tool to query any specified DNS server for a DNS record. The queried DNS server can be a root DNS server, a top -level-domain DNS server, an authoritative DNS server, or an intermediate DNS server (see the textbook for definitions of these terms). To accomplish this task, nslookupsends a DNS query to the specified DNS server, receives a DNS reply from thatsame DNS server, and displays the result.

The above screenshot shows the results of three independent nslookup commands (displayed in the Windows Command Prompt). In this example, the client host is using Windows 7 and is connected to an ISP using ADSL modem. When running nslookup, if no DNS server is specified, then nslookupsends the query to the default DNS server, which in this case is 192.168.29.2. Consider the first command:

- nslookup  www.mit.edu

In words, this command is saying “please send me the IP address for the host www.mit.edu”. As shown in the screenshot, the response from this command provides two pieces of information: (1) the name and IP address of the local DNS server that provides the answer; and (2) the answer itself, which is the host name and IP address (of a CDN node hosting the site) www.mit.edu. Although the response came from the local DNS server, it is quite possible that this local DNS server iteratively contacted several other DNS servers to get the answer, as described in Section 2.5 of the textbook.

Now consider the second command:

- nslookup  –type=NS  mit.edu

In this example, we have provided the option “-type=NS” and the domain “mit.edu”. This causes nslookup to send a query for a type-NS record to the default local DNS server. In words, the query is saying, “please send me the host names of the authoritative DNS servers for mit.edu”. (When the –type option is not used, nslookup uses the default, which is to queryfor type A records.) The answer, displayed in the above screenshot, first indicates the DNS server that is providing the answer (which is the default local DNS server) along with MIT nameservers (again hosted on Akamai CDN). Each of these servers is indeed an authoritative DNS server for the hosts on the MIT campus. However, nslookup also indicates that the answer is “non -authoritative,” meaning that this answer came from the cache of some server rather than from an authoritative MIT DNS server. Note that the answer does not include the IP addresses of the authoritative DNS servers for MIT.

Now finally consider the third command:

- nslookup  www.mit.edu  8.8.8.8

In this example, we indicate that we want the query sent to the Google public DNS server (well known IP 8.8.8.8) rather than to the default local DNS server (192.168.29.2). Thus, the query and reply transaction takes place directly between our querying host and Google public DNS server. In this example, the Google DNS server provided the IP address of the host www.mit.edu that is similar to the response in first nslookup.

Now that we have gone through a few illustrative examples, you are perhaps wondering about the general syntax of nslookup commands. The syntax is:

nslookup  –option1  –option2  host-to-find  dns-server

In general, nslookup can be run with zero, one, two or more options. And as we have seen in the above examples, the dns-server is optional as well; if it is not supplied, the query is sent to the default local DNS server.

You can also run the nslookup in interactive mode. Simply type nslookup and press enter. Now at the > prompt, type help to see the list of all options available.

- Introduction to IPCONFIG

ipconfig (for Windows) andifconfig(for Linux/Unix) are among the most useful littleutilities in your host, especially for debugging network issues. Here we’ll only describe ipconfig, although the Linux/Unixifconfigis very similar. ipconfig can be used to showyour current TCP/IP information, including your address, DNS server addresses, adapter type and so on. For example, if you can get all this information about your host simply by entering:

- Ipconfig /all

into the Command Prompt, as shown in the following screenshot.

ipconfig is also very useful for managing the DNS information stored in your host. InSection 2.5 we learned that a host can cache DNS records it recently obtained. To see these cached records, after the prompt C:\> provide the following command:

- ipconfig  /displaydns

Each entry shows the remaining Time to Live (TTL) in seconds. To clear the cache, enter:

- ipconfig  /flushdns

Flushing the DNS cache clears all entries.

Steps for performing this lab:

Exercise 01: nslookup

Now that we have provided an overview of nslookup, it is time for you to test drive it yourself. You have to use 8.8.8.8 as your local DNS server for all these exercises. Do the following (and write down the results):

1.1 Run nslookup to obtain the IP address of the Web server hosting www.seecs.edu.pk.

1.2 Run nslookup to determine the authoritative DNS servers for domain seecs.edu.pk. Provide both the names of these DNS servers and also the IP addresses of one of the DNS servers.

1.3 Run nslookup to determine the mail servers for seecs.edu.pk. Provide both the names of these Mail servers and also the IP address of one of these Mail servers.

1.4 Query the public DNS service provided by Google at 8.8.8.8 to query for the IPv6 address of www.seecs.edu.pk. Provide the IPv6 address. Note how this address is different from the IPv4 addresses that you were getting for the previous questions.

IPv4 is a 32-bit numeric address and is written in decimals as four separated periods. In example above the ipv4 address of seecs is 111.68.101.54. While ipv6 address is 128-bit ip address written in hexadecimal and separated by colons. In example above the ipv6 address of seecs is 2400:fc00:8070::30

Exercise 02: Tracing DNS with Wireshark (while using browser)

Now that we are familiar with nslookup and ipconfig, we’re ready to get down to some serious business. Let’s first capture the DNS packets that are generated by ordinary Web-surfing activity.

- Use ipconfig to empty the DNS cache in your host.

- Open your browser and empty your browser cache. (With Internet Explorer, go to Tools menu and select Internet Options; then in the General tab select Delete Files.)

- Open Wireshark and enter “ip.addr == your_IP_address” into the filter, where you obtain your_IP_address with ipconfig. This filter removes all packets that neither originate nor are destined to your host.

- Start packet capture in Wireshark.

- With your browser, visit the Web page: http://www.cse.unsw.edu.au

- Stop packet capture.

Answer the following questions:

  - Locate the DNS query and response messages. Are these sent over UDP or TCP (i.e., what transport layer protocol is being used)?

As see in the above diagram DNS uses UDP protocol. It makes sense because UDP is fast and we need to resolve the host configurations as quickly as possible to give smooth internet experience.

  - To what IP address is the DNS query message sent? Use ipconfig to determine the IP address of your local DNS server. Are these two IP addresses the same?

As seen from diagrams the ip addresses of local dns server are same.

2.3What is contained in the flag field of your DNS request and response? Explain each set bit.

First bit is zero it means that this a query. Next 4 bits are zeros meaning that it’s a standard query. The next bit is zero means that message is not truncated. The next bit is 1 showing that recursion is desired. And last two zeros show non-authenticated data.

2.4Find out the IP address of at-least one ROOT name server.

2.5Find out the fully qualified domain name for IP address 149.171.96.2